| # Assignment 7 # Advance HPC Lab |  |
| --- | --- |
| # PRATIK ABHANG | # 260740150018 |
| # AKANKSHA JADHAO | # 260740150004 |

# 1. Aim

To understand NVIDIA GPU architecture generations, describe the main CUDA concepts, and run a simple CUDA program.

# 2. NVIDIA GPU architecture

A GPU contains multiple Streaming Multiprocessors (SMs). An SM is a basic processing building block and can contain CUDA cores, Special Function Units (SFUs), registers, and L1 cache/shared memory. The GPU also uses cache and global memory to supply data to its processing units.

CUDA is NVIDIA’s platform and programming model for general-purpose GPU computing. In the host-device model, the CPU is the host and the GPU is the device. The CPU manages the program and launches GPU kernels for compute-intensive work.

# 3. Architecture generations and improvements

The following summary follows the topics and specifications shown in the supplied NVIDIA GPU microarchitectures slides. Exact core counts depend on the specific GPU model; figures below are the slide’s stated examples, not universal values for every chip in a generation.

| Generation | Slide specifications / features | Improvement or emphasis shown |
| --- | --- | --- |
| Fermi | GF100 SM: 32 CUDA cores; 512 CUDA cores in the described GF100 GPU; configurable shared memory/L1 cache (48 KB shared + 16 KB L1, or 16 KB shared + 48 KB L1). | Introduced an SM L1 cache and configurable on-chip memory in the example described. |
| Kepler | 192 single-precision CUDA cores, 64 double-precision units, 32 SFUs, 32 load/store units, 4 warp schedulers; 48 KB read-only data cache; 64/128 KB L1/shared memory as shown. | More processing units per SM in the slide’s design; multiple warp schedulers and read-only data cache. |
| Pascal | Two processing blocks per SM in the slide; each block lists 32 single-precision CUDA cores, 16 FP64 CUDA cores, one instruction buffer, one warp scheduler, and two dispatch units. | The slide emphasizes a split processing-block design and FP64 units. |
| Volta | Four processing blocks; each block lists 16 FP32, 8 FP64, 16 INT32 cores, 2 mixed-precision Tensor Cores, L0 instruction cache, one warp scheduler, one dispatch unit, and a 64 KB register file. | Adds Tensor Cores for mixed-precision workloads and separates more execution functions. |
| Ampere | Each processing block in the slide lists 32 FP32, 16 FP64, 64 INT32 cores, Tensor Cores, L0 instruction cache, one warp scheduler, one dispatch unit, and a 128 KB register file. | The slide shows increased FP32/INT32 resources and a larger register file compared with its Volta block example. |
| Hopper (H100) | The slide lists 128 FP32, 64 FP64, 64 INT32 cores, 4 fourth-generation Tensor Cores per SM, 60 MB L2 cache, and a 264 KB register file. | Highlights newer Tensor Cores, a large L2 cache, and a larger register file in the slide’s specification. |
| Ada Lovelace | The slide lists 128 FP32, 64 INT32 cores, 4 fourth-generation Tensor Cores per SM, 128 KB L1 cache, one warp scheduler, one dispatch unit, a 264 KB register file, and third-generation RT cores. | Highlights Tensor Cores and ray-tracing hardware for graphics workloads. |

The presentation also names Tesla, Maxwell, and Turing in its generation list, but the supplied slides do not give detailed specifications for those generations. They are therefore listed here without inventing numeric comparisons.

Overall, the slides show a progression from conventional CUDA execution resources and cache/shared-memory arrangements toward more specialized hardware such as Tensor Cores, larger register files/cache resources in the examples, and RT cores in Ada Lovelace. A fair performance comparison requires testing actual GPU models with the same workload.

# 4. CUDA basics

CUDA: Compute Unified Device Architecture; enables general-purpose GPU computing using CUDA C/C++. CUDA source files commonly use the .cu extension.

Host and device: Host = CPU; device = GPU.

Kernel: A GPU function, commonly declared with __global__, that is launched by the host.

Thread: Smallest execution unit; threadIdx identifies a thread within its block.

Warp: A group of 32 threads, described in the slides as a basic GPU scheduling unit.

Block: A group of threads; threads in a block can cooperate using shared memory. blockIdx identifies the block and blockDim gives its dimensions.

Grid: A collection of blocks launched for a kernel.

Execution hierarchy: Kernel launch → Grid → Block → Warp → Thread.

cudaDeviceSynchronize(): Makes the host wait until previously launched GPU work has completed.

cudaMemcpy(): Copies data between host and device memory.

# 5. CUDA environment setup

A CUDA Toolkit installation provides nvcc, libraries, and development tools. A compatible host compiler is also required.

nvcc --version

gcc --version

nvcc hello_world.cu -o hello_world

./hello_world

nvcc vector_add.cu -o vector_add

./vector_add

Run these commands on a system with an NVIDIA GPU, a compatible NVIDIA driver, and CUDA Toolkit installed. The Linux run commands shown here follow the training slides.

# 6. Program 1: CUDA Hello World

This example follows the Hello World program shown on page 39 of the Introduction to CUDA slides. It launches one block containing five threads.

#include <stdio.h>
#include <cuda_runtime.h>

__global__ void hello() {
    printf("Hello from GPU thread %d!\n", threadIdx.x);
}

int main() {
    hello<<<1, 5>>>();
    cudaError_t err = cudaGetLastError();
    if (err != cudaSuccess) {
        printf("Kernel launch error: %s\n", cudaGetErrorString(err));
        return 1;
    }

    err = cudaDeviceSynchronize();
    if (err != cudaSuccess) {
        printf("CUDA execution error: %s\n", cudaGetErrorString(err));
        return 1;
    }

    return 0;
}

# 7. Program 2: CUDA vector addition (extra practice)

This additional example demonstrates a simple GPU calculation. Each thread adds one pair of array elements. It is included as practical supporting work; it is not explicitly specified as a required task in the supplied slides.

#include <stdio.h>
#include <cuda_runtime.h>

__global__ void vectorAdd(const int *a, const int *b, int *c, int n) {
    int i = blockIdx.x * blockDim.x + threadIdx.x;
    if (i < n) {
        c[i] = a[i] + b[i];
    }
}

int main() {
    const int n = 8;
    int h_a[8] = {1, 2, 3, 4, 5, 6, 7, 8};
    int h_b[8] = {10, 20, 30, 40, 50, 60, 70, 80};
    int h_c[8];

    int *d_a = NULL, *d_b = NULL, *d_c = NULL;
    size_t bytes = n * sizeof(int);

    if (cudaMalloc(&d_a, bytes) != cudaSuccess ||
        cudaMalloc(&d_b, bytes) != cudaSuccess ||
        cudaMalloc(&d_c, bytes) != cudaSuccess) {
        printf("Device memory allocation failed.\n");
        return 1;
    }

    cudaMemcpy(d_a, h_a, bytes, cudaMemcpyHostToDevice);
    cudaMemcpy(d_b, h_b, bytes, cudaMemcpyHostToDevice);

    int threadsPerBlock = 256;
    int blocks = (n + threadsPerBlock - 1) / threadsPerBlock;
    vectorAdd<<<blocks, threadsPerBlock>>>(d_a, d_b, d_c, n);

    cudaError_t err = cudaGetLastError();
    if (err == cudaSuccess) err = cudaDeviceSynchronize();
    if (err != cudaSuccess) {
        printf("CUDA error: %s\n", cudaGetErrorString(err));
        cudaFree(d_a); cudaFree(d_b); cudaFree(d_c);
        return 1;
    }

    cudaMemcpy(h_c, d_c, bytes, cudaMemcpyDeviceToHost);
    printf("Vector addition result: ");
    for (int i = 0; i < n; i++) printf("%d ", h_c[i]);
    printf("\n");

    cudaFree(d_a); cudaFree(d_b); cudaFree(d_c);
    return 0;
}

# 8. Expected output

Hello World program: five lines, one from each GPU thread. The order of the five lines may vary because threads execute in parallel.

Example:
Hello from GPU thread 0!
Hello from GPU thread 1!
Hello from GPU thread 2!
Hello from GPU thread 3!
Hello from GPU thread 4!

Vector addition program: Vector addition result: 11 22 33 44 55 66 77 88

# 9. Complete benchmarking practical: matrix multiplication

The supplied CUDA slides describe benchmarking with matrix multiplication: increase the matrix size, measure CPU-only performance (called “Gold Performance” in the slides), measure GPU performance, and repeat. The source material does not provide measured timings, so results must be collected on the computer used for testing.

Save the following program as matrix_benchmark.cu. It compares a simple CPU implementation with a basic CUDA GPU implementation.

#include <stdio.h>
#include <stdlib.h>
#include <cuda_runtime.h>
#include <time.h>

#define IDX(r, c, n) ((r) * (n) + (c))

__global__ void matMulGPU(const float *A, const float *B, float *C, int n) {
    int col = blockIdx.x * blockDim.x + threadIdx.x;
    int row = blockIdx.y * blockDim.y + threadIdx.y;
    if (row < n && col < n) {
        float sum = 0.0f;
        for (int k = 0; k < n; k++)
            sum += A[IDX(row, k, n)] * B[IDX(k, col, n)];
        C[IDX(row, col, n)] = sum;
    }
}

void matMulCPU(const float *A, const float *B, float *C, int n) {
    for (int i = 0; i < n; i++)
        for (int j = 0; j < n; j++) {
            float sum = 0.0f;
            for (int k = 0; k < n; k++)
                sum += A[IDX(i, k, n)] * B[IDX(k, j, n)];
            C[IDX(i, j, n)] = sum;
        }
}

double elapsed_ms(clock_t start, clock_t end) {
    return 1000.0 * (double)(end - start) / CLOCKS_PER_SEC;
}

int main(int argc, char **argv) {
    int n = (argc > 1) ? atoi(argv[1]) : 256;
    if (n < 1) {
        printf("Matrix size must be positive.\\n");
        return 1;
    }
    size_t bytes = (size_t)n * n * sizeof(float);
    float *A = (float *)malloc(bytes), *B = (float *)malloc(bytes);
    float *Ccpu = (float *)malloc(bytes), *Cgpu = (float *)malloc(bytes);
    if (!A || !B || !Ccpu || !Cgpu) {
        printf("Host memory allocation failed.\\n");
        free(A); free(B); free(Ccpu); free(Cgpu);
        return 1;
    }
    for (int i = 0; i < n*n; i++) {
        A[i] = (float)((i % 7) + 1) / 7.0f;
        B[i] = (float)((i % 5) + 1) / 5.0f;
    }

    clock_t cpu_start = clock();
    matMulCPU(A, B, Ccpu, n);
    clock_t cpu_end = clock();
    double cpu_ms = elapsed_ms(cpu_start, cpu_end);

    float *dA = NULL, *dB = NULL, *dC = NULL;
    cudaError_t err = cudaMalloc(&dA, bytes);
    if (err == cudaSuccess) err = cudaMalloc(&dB, bytes);
    if (err == cudaSuccess) err = cudaMalloc(&dC, bytes);
    if (err != cudaSuccess) {
        printf("CUDA allocation failed: %s\\n", cudaGetErrorString(err));
        if (dA) cudaFree(dA); if (dB) cudaFree(dB); if (dC) cudaFree(dC);
        free(A); free(B); free(Ccpu); free(Cgpu);
        return 1;
    }

    cudaMemcpy(dA, A, bytes, cudaMemcpyHostToDevice);
    cudaMemcpy(dB, B, bytes, cudaMemcpyHostToDevice);
    dim3 threads(16, 16);
    dim3 blocks((n + threads.x - 1) / threads.x,
                (n + threads.y - 1) / threads.y);

    // Warm-up and synchronize before timing.
    matMulGPU<<<blocks, threads>>>(dA, dB, dC, n);
    err = cudaGetLastError();
    if (err == cudaSuccess) err = cudaDeviceSynchronize();
    if (err != cudaSuccess) {
        printf("CUDA kernel failed: %s\\n", cudaGetErrorString(err));
        cudaFree(dA); cudaFree(dB); cudaFree(dC);
        free(A); free(B); free(Ccpu); free(Cgpu);
        return 1;
    }

    cudaEvent_t start, stop;
    cudaEventCreate(&start); cudaEventCreate(&stop);
    cudaEventRecord(start);
    matMulGPU<<<blocks, threads>>>(dA, dB, dC, n);
    cudaEventRecord(stop);
    cudaEventSynchronize(stop);
    float gpu_kernel_ms = 0.0f;
    cudaEventElapsedTime(&gpu_kernel_ms, start, stop);
    cudaMemcpy(Cgpu, dC, bytes, cudaMemcpyDeviceToHost);

    int correct = 1;
    for (int i = 0; i < n*n; i++) {
        float diff = Ccpu[i] - Cgpu[i];
        if (diff < -0.001f || diff > 0.001f) { correct = 0; break; }
    }

    printf("Matrix size: %d x %d\\n", n, n);
    printf("CPU multiplication time: %.3f ms\\n", cpu_ms);
    printf("GPU kernel time (excludes transfers): %.3f ms\\n", gpu_kernel_ms);
    printf("Result check: %s\\n", correct ? "PASS" : "FAIL");
    if (gpu_kernel_ms > 0.0f)
        printf("CPU time / GPU kernel time: %.2f x (kernel-only comparison)\\n",
               cpu_ms / gpu_kernel_ms);

    cudaEventDestroy(start); cudaEventDestroy(stop);
    cudaFree(dA); cudaFree(dB); cudaFree(dC);
    free(A); free(B); free(Ccpu); free(Cgpu);
    return correct ? 0 : 1;
}

# 10. How to run the matrix benchmark

nvcc matrix_benchmark.cu -o matrix_benchmark

./matrix_benchmark 128

./matrix_benchmark 256

./matrix_benchmark 512

./matrix_benchmark 1024

Record the output for every size. CPU time and GPU kernel time are not a perfectly equivalent end-to-end comparison: the GPU number excludes memory transfers, as stated by the program. For a fair end-to-end comparison, separately measure host-to-device copies, kernel execution, and device-to-host copy, and report the total GPU time too.

# 11. Final observation table to complete after running

| N × N | CPU time (ms) | GPU kernel (ms) | Speed ratio | Result check |
| --- | --- | --- | --- | --- |
| 128 × 128 | Enter measured value | Enter measured value | CPU/GPU | PASS/FAIL |
| 256 × 256 | Enter measured value | Enter measured value | CPU/GPU | PASS/FAIL |
| 512 × 512 | Enter measured value | Enter measured value | CPU/GPU | PASS/FAIL |
| 1024 × 1024 | Enter measured value | Enter measured value | CPU/GPU | PASS/FAIL |