Assignment 7

PNR: 260740150018

Name: PRATIK SUNIL ABHANG

1. Standard ACL - Block One Host

Question: Configure a Standard ACL to block a particular host from accessing all connected devices of the second router.

Commands

! Configure on Router1 access-list 10 deny host 192.168.10.10 access-list 10 permit any interface se1/0 ip access-group 10 in exit end write

Check / Test

show access-lists show ip interface se1/0 ! From PC0: ping 192.168.30.10 ping 192.168.40.10

Screenshot

2. Extended ACL - Block One Network

Question: Configure an Extended ACL to block one entire network from accessing another network.

Commands

! Configure on Router0 access-list 110 deny ip 192.168.10.0 0.0.0.255 192.168.30.0 0.0.0.255 access-list 110 permit ip any any interface fa0/0 ip access-group 110 in exit end write

Check / Test

show access-lists 110 show ip interface fa0/0 ! From a PC in 192.168.10.0/24: ping 192.168.30.10 ! Other destination test: ping 192.168.40.10

Screenshot

3. Extended ACL - Block Telnet and HTTP

Question: Configure an ACL to block services Telnet and HTTP between two networks.

Commands

! Configure on Router0 access-list 120 deny tcp 192.168.10.0 0.0.0.255 192.168.30.0 0.0.0.255 eq 23 access-list 120 deny tcp 192.168.10.0 0.0.0.255 192.168.30.0 0.0.0.255 eq 80 access-list 120 permit ip any any interface fa0/0 ip access-group 120 in exit end write

Check / Test

show access-lists 120 show ip interface fa0/0 ! Telnet test: telnet 192.168.30.10 ! HTTP: open the destination Server web page ! Other traffic should work: ping 192.168.30.10

Screenshot

1. Static NAT

Question: Configure Static NAT on two routers so that an internal private IP address is translated to a fixed public IP address.

Commands

! Router0 interface fa0/0 ip nat inside exit interface fa0/1 ip nat inside exit interface se1/0 ip nat outside exit ip nat inside source static 192.168.10.10 100.0.0.10 ip route 200.0.0.0 255.255.255.0 10.0.0.2 ! Router1 interface fa0/0 ip nat inside exit interface fa0/1 ip nat inside exit interface se1/0 ip nat outside exit ip nat inside source static 192.168.30.10 200.0.0.10 ip route 100.0.0.0 255.255.255.0 10.0.0.1 end write

Check / Test

show ip nat translations show ip nat statistics ! Confirm mappings: ! R0: 192.168.10.10 <-> 100.0.0.10 ! R1: 192.168.30.10 <-> 200.0.0.10

Screenshot

2. Dynamic NAT

Question: Configure Dynamic NAT on two routers so that a pool of public IP addresses is used to translate multiple private IP addresses.

Commands

! Router0 interface fa0/0 ip nat inside exit interface fa0/1 ip nat inside exit interface se1/0 ip nat outside exit access-list 1 permit 192.168.10.0 0.0.0.255 access-list 1 permit 192.168.20.0 0.0.0.255 ip nat pool R1POOL 100.0.0.10 100.0.0.20 netmask 255.255.255.0 ip nat inside source list 1 pool R1POOL ! Router1 interface fa0/0 ip nat inside exit interface fa0/1 ip nat inside exit interface se1/0 ip nat outside exit access-list 2 permit 192.168.30.0 0.0.0.255 access-list 2 permit 192.168.40.0 0.0.0.255 ip nat pool R2POOL 200.0.0.10 200.0.0.20 netmask 255.255.255.0 ip nat inside source list 2 pool R2POOL end write

Check / Test

show access-lists 1 show access-lists 2 show ip nat translations show ip nat statistics ! Generate traffic from several inside PCs first.

Screenshot

3. PAT (NAT Overload)

Question: Configure PAT (Port Address Translation) on two routers so that many private IP addresses share a single public IP address using different ports.

Commands

! Router0 interface fa0/0 ip nat inside exit interface fa0/1 ip nat inside exit interface se1/0 ip nat outside exit access-list 11 permit 192.168.10.0 0.0.0.255 access-list 11 permit 192.168.20.0 0.0.0.255 ip nat inside source list 11 interface se1/0 overload ! Router1 interface fa0/0 ip nat inside exit interface fa0/1 ip nat inside exit interface se1/0 ip nat outside exit access-list 12 permit 192.168.30.0 0.0.0.255 access-list 12 permit 192.168.40.0 0.0.0.255 ip nat inside source list 12 interface se1/0 overload end write

Check / Test

show access-lists 11 show access-lists 12 show ip nat translations show ip nat statistics ! Generate simultaneous traffic from several PCs.

Screenshot

1. RIPng for IPv6

Question: Configure RIP for IPv6 on two routers so that both routers exchange IPv6 routing information.

Commands

! Router0 ipv6 unicast-routing interface fa0/0 ipv6 address 2001:DB8:1::1/64 no shutdown exit interface se1/0 ipv6 address 2001:DB8:12::1/64 clock rate 64000 no shutdown exit ipv6 router rip RIPNG exit interface fa0/0 ipv6 rip RIPNG enable exit interface se1/0 ipv6 rip RIPNG enable exit ! Router1 ipv6 unicast-routing interface fa0/0 ipv6 address 2001:DB8:2::1/64 no shutdown exit interface se1/0 ipv6 address 2001:DB8:12::2/64 no shutdown exit ipv6 router rip RIPNG exit interface fa0/0 ipv6 rip RIPNG enable exit interface se1/0 ipv6 rip RIPNG enable exit end write

Check / Test

show ipv6 interface brief show ipv6 route show ipv6 protocols ! Router0 should learn 2001:DB8:2::/64 as R. ! Router1 should learn 2001:DB8:1::/64 as R. ! From PC0: ping 2001:DB8:2::10

Screenshot

2. EIGRP Routing for IPv6

Question: Two routers are connected in a line; each router has two switches, and each switch has two PCs. Configure EIGRP Routing. Assign the same AS number to the routers. Check whether the PCs can communicate.

Commands

! Router0 ipv6 unicast-routing interface fa0/0 ipv6 address 2001:DB8:1::1/64 ipv6 eigrp 100 no shutdown exit interface fa0/1 ipv6 address 2001:DB8:2::1/64 ipv6 eigrp 100 no shutdown exit interface se1/0 ipv6 address 2001:DB8:12::1/64 ipv6 eigrp 100 clock rate 64000 no shutdown exit ipv6 router eigrp 100 eigrp router-id 1.1.1.1 no shutdown exit ! Router1 ipv6 unicast-routing interface fa0/0 ipv6 address 2001:DB8:3::1/64 ipv6 eigrp 100 no shutdown exit interface fa0/1 ipv6 address 2001:DB8:4::1/64 ipv6 eigrp 100 no shutdown exit interface se1/0 ipv6 address 2001:DB8:12::2/64 ipv6 eigrp 100 no shutdown exit ipv6 router eigrp 100 eigrp router-id 2.2.2.2 no shutdown exit end write

Check / Test

show ipv6 interface brief show ipv6 eigrp neighbors show ipv6 route ! Look for R0/R1 as EIGRP neighbors. ! Remote routes should be marked D. ! From PC0: ping 2001:DB8:3::10 ping 2001:DB8:4::10

Screenshot

3. OSPFv3 for IPv6

Question: Two routers are connected in a line; each router has two switches, and each switch has two PCs. Configure OSPF. Make the serial interface a backbone area and the FastEthernet ports non-backbone areas with different area numbers. Check whether the PCs can communicate.

Commands

! Router0 ipv6 unicast-routing interface fa0/0 ipv6 address 2001:DB8:1::1/64 no shutdown exit interface fa0/1 ipv6 address 2001:DB8:2::1/64 no shutdown exit interface se1/0 ipv6 address 2001:DB8:12::1/64 clock rate 64000 no shutdown exit ipv6 router ospf 1 router-id 1.1.1.1 exit interface fa0/0 ipv6 ospf 1 area 1 exit interface fa0/1 ipv6 ospf 1 area 2 exit interface se1/0 ipv6 ospf 1 area 0 exit ! Router1 ipv6 unicast-routing interface fa0/0 ipv6 address 2001:DB8:3::1/64 no shutdown exit interface fa0/1 ipv6 address 2001:DB8:4::1/64 no shutdown exit interface se1/0 ipv6 address 2001:DB8:12::2/64 no shutdown exit ipv6 router ospf 1 router-id 2.2.2.2 exit interface fa0/0 ipv6 ospf 1 area 3 exit interface fa0/1 ipv6 ospf 1 area 4 exit interface se1/0 ipv6 ospf 1 area 0 exit end write

Check / Test

show ipv6 interface brief show ipv6 ospf neighbor show ipv6 ospf interface show ipv6 route ! Serial1/0 must be Area 0 on both routers. ! Each FastEthernet LAN must have a different non-backbone area. ! From PC0: ping 2001:DB8:3::10 ping 2001:DB8:4::10

Screenshot

Assignment 8

PNR: 260740150018

Name: PRATIK SUNIL ABHANG

Question 1: VLAN

Configure VLAN 10 and VLAN 20 on Switch1

Commands:

enable
configure terminal
vlan 10
name a
exit
vlan 20
name b
exit
interface fa0/2
switchport mode access
switchport access vlan 10
exit

interface fa0/3
switchport mode access
switchport access vlan 20
end

Configure VLAN 10 and VLAN 20 on Switch2

Objective: Create the same VLANs and assign the PC ports.

Commands:

enable
configure terminal
vlan 10
name a
exit
vlan 20
name b
exit
interface fa0/2
switchport mode access
switchport access vlan 10
exit
interface fa0/3
switchport mode access
switchport access vlan 20
end

Configure PC IP addresses and test VLAN communication

Commands:

PC1 (VLAN 10): 192.168.10.10 / 255.255.255.0
PC2 (VLAN 20): 192.168.20.10 / 255.255.255.0
PC3 (VLAN 10): 192.168.10.20 / 255.255.255.0
PC4 (VLAN 20): 192.168.20.20 / 255.255.255.0

From PC1 Command Prompt:
ping 192.168.10.20    ← should work (same VLAN)
ping 192.168.20.20    ← should fail (different VLAN, no router)

Question 2: VTP

Configure VTP Server and trunk on Switch1

Commands:

enable
configure terminal
interface fa0/1
switchport mode trunk
switchport trunk allowed vlan 10,20
exit
vtp domain company
vtp mode server
vlan 10
name a
exit
vlan 20
name b
exit
end
show vtp status

Configure VTP Client on Switch2

Commands:

enable
configure terminal
interface fa0/1
switchport mode trunk
switchport trunk allowed vlan 10,20
exit
vtp domain company
vtp mode client
end
show vtp status

.

Question 7: Verify VTP configuration

Objective: Check that the VTP mode and domain name are correct and that the VLAN information is available.

Commands:

show vtp status
show vlan brief
show interfaces trunk

Question 3: STP

Create the STP topology

Configure Switch1 as the STP Root Bridge

Commands:

enable
configure terminal
spanning-tree vlan 1 priority 4096
end
show spanning-tree

Question 10: Verify the STP result on the switches

Commands:

On each switch:
enable
show spanning-tree