PNR: 260740150018

Name: PRATIK SUNIL ABHANG

Set enable password and enable secret password on the router and verify encryption. Also set a console password and access the router from a PC.

# 1. Topology and Connections

Router: Cisco 2911

Switch: Cisco 2960

PC 1: LAN PC with IP 192.168.10.2

PC 2: LAN PC with IP 192.168.10.3

PC 3: Console PC connected to the router Console port with a console cable

Router-to-switch and switch-to-PC Ethernet links use Copper Straight-Through cables

# 2. Open the Router Through the Console

Use the PC connected to the router with the blue console cable.

PC → Desktop → Terminal → accept the default serial settings → OK.

At the router prompt:

Router> enable
Router# configure terminal

# 3. Set the Enable Password and Enable Secret

Router(config)# enable password cisco123
Router(config)# enable secret cisco1234

Enable password = cisco123. Enable secret = cisco1234. If both are configured, the enable secret takes precedence when entering privileged EXEC mode.

# 4. Set the Console Password

Router(config)# line console 0
Router(config-line)# password console123
Router(config-line)# login
Router(config-line)# exit

The login command is required so the console requests the configured password.

# 5. Configure the Router LAN Interface

Router(config)# interface fastethernet 0/0
Router(config-if)# ip address 192.168.10.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# exit

# 6. Verify the Password Configuration and Encryption

Router# show running-config

# 7. Configure the LAN PCs

For PC 1 (the PC labeled 192.168.10.2):

IP Address:      192.168.10.2
Subnet Mask:     255.255.255.0
Default Gateway: 192.168.10.1

For PC 2 (the PC labeled 192.168.10.3):

IP Address:      192.168.10.3
Subnet Mask:     255.255.255.0
Default Gateway: 192.168.10.1

# 8. Test PC-to-Router Connectivity

From PC 1 or PC 2, open Desktop → Command Prompt and run:

C:\> ping 192.168.10.1

# 9. Configure Telnet on the Router

Telnet access is configured on the VTY lines:

Router> enable
Router# configure terminal
Router(config)# line vty 0 4
Router(config-line)# password telnet123
Router(config-line)# login
Router(config-line)# transport input telnet
Router(config-line)# exit
Router(config)# end

# 10. Access the Router from a PC Using Telnet

Use PC 1 (192.168.10.2) or PC 2 (192.168.10.3). Open Desktop → Command Prompt.

C:\> telnet 192.168.10.1

Password:

telnet123

Router>

# 11. Test the Enable Secret Through Telnet

Router> enable
Password:
cisco1234
Router#

# 12. Final Verification Commands

Router# show running-config
Router# show ip interface brief