PNR: 260740150018

Name: PRATIK SUNIL ABHANG

Question 1: VLAN

Configure VLAN 10 and VLAN 20 on Switch1

Commands:

enable
configure terminal
vlan 10
name a
exit
vlan 20
name b
exit
interface fa0/2
switchport mode access
switchport access vlan 10
exit

interface fa0/3
switchport mode access
switchport access vlan 20
end

Configure VLAN 10 and VLAN 20 on Switch2

Objective: Create the same VLANs and assign the PC ports.

Commands:

enable
configure terminal
vlan 10
name a
exit
vlan 20
name b
exit
interface fa0/2
switchport mode access
switchport access vlan 10
exit
interface fa0/3
switchport mode access
switchport access vlan 20
end

Configure PC IP addresses and test VLAN communication

Commands:

PC1 (VLAN 10): 192.168.10.10 / 255.255.255.0
PC2 (VLAN 20): 192.168.20.10 / 255.255.255.0
PC3 (VLAN 10): 192.168.10.20 / 255.255.255.0
PC4 (VLAN 20): 192.168.20.20 / 255.255.255.0

From PC1 Command Prompt:
ping 192.168.10.20    ← should work (same VLAN)
ping 192.168.20.20    ← should fail (different VLAN, no router)

Question 2: VTP

Configure VTP Server and trunk on Switch1

Commands:

enable
configure terminal
interface fa0/1
switchport mode trunk
switchport trunk allowed vlan 10,20
exit
vtp domain company
vtp mode server
vlan 10
name a
exit
vlan 20
name b
exit
end
show vtp status

Configure VTP Client on Switch2

Commands:

enable
configure terminal
interface fa0/1
switchport mode trunk
switchport trunk allowed vlan 10,20
exit
vtp domain company
vtp mode client
end
show vtp status

.

Question 7: Verify VTP configuration

Objective: Check that the VTP mode and domain name are correct and that the VLAN information is available.

Commands:

show vtp status
show vlan brief
show interfaces trunk

Question 3: STP

Create the STP topology

Configure Switch1 as the STP Root Bridge

Commands:

enable
configure terminal
spanning-tree vlan 1 priority 4096
end
show spanning-tree

Question 10: Verify the STP result on the switches

Commands:

On each switch:
enable
show spanning-tree